PCM System Design & Analysis Project Report

Presented by:

Mario Makhlouta A2220007

Antoine Marie Lahoud A2220266

Miguel Nawfal A2212183

Presented to:

Dr. Olga Yaacoub

Telecommunication Lab

Faculty of Engineering

University of Balamand

Contents

Figures	2

1. Introduction	3

2. Objective	3

3. PCM Theory	3

4. PCM System Overview	4

5. Design Specifications	5

6. Simulink Implementation	6

7. Results and Discussion	9

8. Quantization Noise Analysis	15

9. Performance Evaluation	16

10. Advantages and Disadvantages	17

11. Future Work	17

12. Conclusion	18

# Figures

Figure 1: PCM System Overview	5

Figure 2: Simulink Solution	8

Figure 3: Input Signal	9

Figure 4: Sampled Signal	10

Figure 5: Quantized Signal	11

Figure 6: PCM Bits	12

Figure 7: DAC Output	13

Figure 8: Reconstructed Signal	14

Figure 9: Input vs Reconstructed	15

# 1. Introduction

Digital communication systems convert analog signals into digital form for reliable transmission. Unlike analog communication, which uses continuous signals, digital communication represents information using discrete values (0 and 1).

Pulse Code Modulation (PCM) is one of the most important techniques used in digital communication systems. It converts an analog signal into a digital bit stream that can be transmitted efficiently over communication channels.

In this project, a complete PCM system is designed and implemented using MATLAB Simulink. The system demonstrates all stages of PCM including sampling, quantization, encoding, decoding, and signal reconstruction.

# 2. Objective

The objective of this project is:

- To design a complete PCM modulator and demodulator using MATLAB Simulink

- To observe the signal at each stage using scopes

- To analyze quantization noise and its effects

- To evaluate system performance using SQNR

- To study the effect of different quantization levels

This directly follows the project requirement described in the assignment.

# 3. PCM Theory

Pulse Code Modulation (PCM) is a technique used to convert analog signals into digital form.

It consists of three main stages:

1. Sampling

The analog signal is measured at regular intervals.

2. Quantization

Each sample is approximated to the nearest discrete amplitude level.

3. Encoding

Each quantized level is converted into binary bits.

Key Characteristics of PCM:

- Sampling frequency must satisfy Nyquist criteria

- Higher quantization levels → better signal quality

- Higher number of bits → higher bandwidth requirement

- Digital transmission improves noise immunity

# 4. PCM System Overview

Transmitter

Analog Signal → Sampler → Quantizer → Encoder → PCM Bitstream

Receiver

PCM Bitstream → Decoder → DAC → Low-Pass Filter → Reconstructed Signal

Figure 1: PCM System Overview

# 5. Design Specifications

| Parameter | Value |
| --- | --- |
| Amplitude | 1 V |
| Message Frequency | 950 Hz |
| Sampling Frequency | 8000 Hz |
| Sampling Time | 0.000125 s |
| Quantization Levels (L) | 16 |
| Bits per sample (n) | 4 |
| Step size (Δ) | 0.125 |
| LPF Cutoff | 2000-2500 Hz |

# 6. Simulink Implementation

Step 1: Model Setup

- Open Simulink → Blank Model

- Set Stop Time = 0.02 s

- Use Fixed-step solver

Step 2: Input Signal

- Add Sine Wave block

- Amplitude = 1

- Frequency = 950 Hz

- Connect to Scope → Input Signal

Step 3: Sampling

- Add Zero-Order Hold

- Sample time = 0.000125 s

- Output → Scope → Sampled Signal

Step 4: Saturation

- Add Saturation block

- Limits = [-1, +1]

- Prevents overload distortion

Step 5: Quantizer

Implement using:

- Gain (1/Δ = 8)

- Round

- Gain (Δ = 0.125)

Output → Scope → Quantized Signal

Step 6: Index Mapping

Use:

- Sum (+1)

- Gain (8)

- Round

- Saturation [0–15]

Step 7: Encoding

- Use Integer to Bit Converter

- Number of bits = 4

- Output → Scope → PCM Bits

Step 8: Decoding

- Use Bit to Integer Converter

Step 9: Reconstruction

Use:

- Gain (0.125)

- Sum (-1)

Step 10: DAC

- Add Zero-Order Hold

- Output → Scope → DAC Output

Step 11: Low-Pass Filter

- Use Transfer Function or LPF block

- Cutoff = 2500 Hz

- Output → Scope → Reconstructed Signal

Figure 2: Simulink Solution

# 7. Results and Discussion

Figure 3: Input Signal

The input signal is a continuous sinusoidal waveform representing the original analog signal. It has a smooth shape with constant amplitude and frequency, serving as the reference for comparison.

Figure 4: Sampled Signal

The sampled signal shows discrete-time samples of the original signal obtained at a sampling frequency of 8 kHz. The waveform appears as a staircase-like signal, confirming proper sampling using Zero-Order Hold.

Figure 5: Quantized Signal

The quantized signal displays discrete amplitude levels, where each sample is approximated to the nearest quantization level. This introduces small errors compared to the original signal, known as quantization noise.

Figure 6: PCM Bits

The PCM bitstream represents the digital encoding of the quantized signal using 4 bits per sample. The signal consists of binary sequences (0s and 1s), confirming successful conversion from analog to digital form.

Figure 7: DAC Output

The DAC output shows a staircase waveform generated from the decoded digital signal. This signal approximates the original waveform but contains sharp transitions due to discrete reconstruction.

Figure 8: Reconstructed Signal

The reconstructed signal is obtained after passing the DAC output through a low-pass filter. It closely resembles the original sinusoidal signal, with minor distortions caused by quantization and filtering effects.

The system successfully converts the analog signal into a digital PCM signal and reconstructs it at the receiver.

Observations

- The sampled signal is discrete in time

- The quantized signal shows amplitude levels

- PCM output is binary

- DAC output is a staircase waveform

- Low-pass filter smooths the signal

Input vs Output Comparison

Figure 9: Input vs Reconstructed

The comparison shows that the reconstructed signal closely follows the original input signal in both shape and frequency. A slight amplitude difference and small distortion are observed due to quantization noise and filter attenuation.

This confirms correct system operation.

# 8. Quantization Noise Analysis

- Quantization Error: e[n]=x[n]-xq[n]

- Noise Power: Pq=Δ212

Types of Quantization Noise:

- Granular Noise

Small error within quantization range

- Overload Distortion

Occurs when signal exceeds allowed range

- Correlated Noise

Produces harmonic distortion in periodic signals

Noise Reduction Methods:

- Increase number of quantization levels

- Avoid overload using saturation

- Use non-uniform quantization (μ-law, A-law)

# 9. Performance Evaluation

Formula:     SQNR=PsPq         SQNRdB=10log⁡10PsPq

For L = 16:

- Δ = 0.125

- Signal power ≈ 0.5

- Noise power ≈ 0.0013

- SQNR≈26 dB

Effect of Quantization Levels

| L | Bits | SQNR |
| --- | --- | --- |
| 8 | 3 | ~20 dB |
| 16 | 4 | ~26 dB |
| 64 | 6 | ~38 dB |
| 256 | 8 | ~50 dB |

  - Increasing bits improves SQNR (~6 dB per bit)

# 10. Advantages and Disadvantages

Advantages:

- High noise immunity

- Suitable for digital systems

- Easy processing and storage

- Supports error correction

Disadvantages:

- Requires high bandwidth

- Quantization noise exists

- More complex system design

# 11. Future Work

- Implement μ-law or A-law companding

- Test using real audio signals

- Add channel noise (AWGN)

- Compare with DPCM and Delta Modulation

# 12. Conclusion

In this project, a complete PCM system was successfully designed and implemented using MATLAB Simulink.

The system demonstrated:

- Sampling

- Quantization

- Encoding and decoding

- Signal reconstruction

The results showed that increasing quantization levels improves signal quality and reduces noise. The reconstructed signal closely matches the original signal, confirming the effectiveness of PCM in digital communication systems.

# References

1. S. Haykin, Digital Communication Systems, 1st Edition. Hoboken, NJ: John Wiley & Sons, 2014

2. J. G. Proakis and M. Salehi, Digital Communications, 5th Edition. New York: McGraw-Hill, 2008

3. MathWorks, "Simulink Documentation — Zero-Order Hold, Quantizer, and Communications Toolbox," MathWorks, 2024. [Online]. Available: https://www.mathworks.com/help/simulink/

4. International Telecommunication Union, "ITU-T Recommendation G.711: Pulse Code Modulation (PCM) of Voice Frequencies," ITU-T, Geneva, 1988 (revised 2000). [Online]. Available: https://www.itu.int/rec/T-REC-G.711/